### 11. Routing Fundamentals & Static Routing

### Routing Fundamentals

### What is Routing?

Routing is the process that routers use to determine the best path for IP packets to travel through a network to reach their destination.

Key Points:

- Routers make routing decisions by consulting a routing table, which contains routes to known destinations.

- When a router receives an IP packet, it refers to its routing table to determine the next-hop or the next destination for the packet.

### Types of Routing

There are two primary methods that routers use to learn and manage routes:

#### 1. Dynamic Routing

- Dynamic Routing Protocols (e.g., OSPF, EIGRP, BGP) are used.

- Routers share routing information automatically, updating their routing tables as the network changes.

- Advantages:

  - Automatically adapts to network changes.

  - Scales well for larger, more complex networks.

- Disadvantages:

  - Requires more CPU, memory, and bandwidth.

  - Can take time to converge after network changes.

#### 2. Static Routing

- Routes are manually configured by a network administrator.

- Used in small or stable networks where paths rarely change.

- Advantages:

  - Simple to configure.

  - Consumes no extra resources (e.g., bandwidth, CPU).

- Disadvantages:

  - Does not adapt to network changes automatically.

  - Labor-intensive for larger networks.

### Routing Decision Process

A route tells the router one of three things:

- Next-Hop Routing:

  - For destination X, forward the packet to next-hop Y.

- Directly Connected:

  - If the destination is on a network directly connected to the router, send the packet to the destination directly.

- Local Delivery:

  - If the destination is the router’s own IP address, the router processes the packet itself (does not forward it).

Routers will choose the most specific matching route.

### WAN and Routing

A Wide Area Network (WAN) spans a large geographical area, often connecting multiple smaller networks (LANs). Routers are crucial in WANs to manage traffic between geographically separated locations.

### Visual Aids

- Routing Table Example: R1# show ip route
 The routing table contains the following for each route:

  - Destination: The network or device to reach.

  - Next-Hop: The next device (or interface) to forward the packet to.

  - Metric: A value that indicates the cost of the route (lower is better).

  - Interface: The router's interface used to send the packet.

- Routing in a WAN:

  - Routers within the WAN maintain routes to other routers and networks.

  - Dynamic routing protocols are often used in large WANs to handle complex topologies.

- WAN Diagram: Visualizes routers and their connections across a large geographical area.

- Routing Table Visualization: Explains how routers reference routes for decision-making.

- Next-Hop Example: Shows how packets are forwarded step-by-step in a network.

### Key Takeaways

- Routing ensures efficient delivery of IP packets across networks.

- Static routing is simple but inflexible; dynamic routing adapts but requires more resources.

- Routers play a pivotal role in WANs, ensuring connectivity over vast geographical areas.

### Static Routing

### Review

#### Switches vs. Routers

- Switches: Operate within a Local Area Network (LAN) to forward traffic between devices on the same network.

- Routers: Operate between LANs, forwarding traffic to destinations across different networks.

### WAN (Wide Area Network)

- A WAN is a network that spans large geographic areas, connecting multiple LANs.

- Routers enable communication between networks across a WAN.

### Static Routing Overview

Static routing involves manually configuring routes on routers. This type of routing is used in small, stable networks or in specific parts of larger networks.

Key Characteristics:

- Manually Configured: A network administrator specifies the route to a destination.

- Simple and Resource-Efficient: No additional CPU or bandwidth is required.

- Inflexible: Routes do not adapt to network changes automatically.

#### Why Use Static Routing?

- It provides greater control over routing decisions.

- It is efficient for predictable, small, or simple network topologies.

### Static Route Configuration

#### Basic Syntax:

ip route <destination-network> <subnet-mask> <next-hop-IP>

Parameters Explained:

- destination-network: The network you want to route packets to.

- subnet-mask: The subnet mask for the destination network.

- next-hop-IP: The IP address of the next router in the path to the destination.

### Static Route with Exit Interface

Another way to configure static routing is by specifying the exit-interface instead of the next-hop IP address.

#### Syntax:

ip route <destination-network> <subnet-mask> <exit-interface>

Example:

ip route 192.168.2.0 255.255.255.0 Serial0/0/0

- In this example, packets destined for 192.168.2.0/24 are sent out the router's Serial0/0/0 interface.

### Default Routes

A default route is used when no other route matches the destination of a packet. It serves as a "catch-all" route.

#### Syntax:

ip route 0.0.0.0 0.0.0.0 <next-hop-IP>

Key Points:

- 0.0.0.0/0 represents any network address.

- The router forwards packets that do not match any specific routes to the default route's next hop.

- Static Route Configuration:

  - Displays examples of static route syntax and implementation.

- Static Route with Exit-Interface:

  - Demonstrates the alternative method of specifying an interface instead of the next-hop IP.

- Default Route:

  - Highlights the importance of the default route for packets without a specific destination.

### Key Takeaways

- Static routes are manually configured and suitable for small or stable networks.

- Using the exit interface in static routing reduces dependency on the next-hop IP address but can have limitations in complex topologies.

- A default route is essential for handling traffic with unknown destinations, especially when connecting to external networks like the internet.