# 17. VLANs: PART 2

## Basic VLAN Topology from Part 1

### Basic VLAN Example:

## What About This Network Topology?

### Network Example with Two Switches (SW1 and SW2):

- Notice:

  - SW1 and SW2 are connected.

  - The Engineering VLAN (VLAN 10) spans two separate locations.

## TRUNK PORTS

### Why Use TRUNK PORTS?

- In small networks, you can use a separate interface for each VLAN to connect switches to switches or switches to routers.

- For larger networks with many VLANs:

  - Using separate interfaces for each VLAN is inefficient and wastes ports.

  - Most routers won’t have enough interfaces for every VLAN.

- Solution: Use TRUNK PORTS to carry traffic for multiple VLANs over a single interface.

### Trunk Port Diagram:

### How Does a Packet Know Which VLAN to Use Over the TRUNK?

#### VLAN TAGGING

- Switches add tags to frames sent over a trunk link.

- This allows the receiving switch to know which VLAN the frame belongs to.

#### Port Types:

- Trunk Port = “Tagged”

- Access Port = “Untagged”

## VLAN TAGGING PROTOCOLS

### Two Main Trunk Protocols:

- ISL (Inter-Switch Link)

  - Old, Cisco proprietary protocol.

  - Rarely used today.

- IEEE 802.1Q (aka “dot1q”)

  - Industry-standard protocol.

  - Most modern switches support 802.1Q exclusively.

#### Ethernet Header with 802.1Q Tag:

### 802.1Q Tag Structure:

- 802.1Q Tag is inserted between the Source and Type/Length fields in the Ethernet frame.

- The tag is 4 bytes (32 bits) long and consists of:

  - TPID (Tag Protocol Identifier):

    - 16 bits (2 bytes).

    - Value: 0x8100 indicates the frame is 802.1Q tagged.

  - TCI (Tag Control Information):

    - Includes three subfields:

      - PCP (Priority Code Point): 3 bits (for Class of Service).

      - DEI (Drop Eligible Indicator): 1 bit.

      - VID (VLAN ID): 12 bits (range 1–4094).

## VLAN RANGES

- VLAN IDs: 1 – 4094

- Reserved VLANs:

  - VLAN 0 and VLAN 4095 (can’t be used).

- Common VLAN Ranges:

  - 1 – 1005: Standard VLANs.

  - 1006 – 4094: Extended VLANs.

NATIVE VLAN

- Definition: VLAN that is not tagged on a trunk link.

- By default, VLAN 1 is the Native VLAN on Cisco switches.

- Best practice: Use a dedicated VLAN for the Native VLAN (e.g., VLAN 99).

- Important that the Native VLAN matches between Switches!

## TRUNK CONFIGURATION

### How to Configure a Trunk Port:

- Select the interface to configure.

- Set the encapsulation type to 802.1Q:
switchport trunk encapsulation dot1q

- Set the interface mode to Trunk:
switchport mode trunk

- Check:
do show interfaces trunk

### Allowing VLANs on a Trunk:

- Use the following command to allow specific VLANs on a trunk:
switchport trunk allowed vlan <vlan_list>

## ROUTER ON A STICK (ROAS)

### What Is ROAS?

- ROAS allows you to route between multiple VLANs using a single physical interface on a router.

- Router uses sub-interfaces for each VLAN.

#### Key Points:

- Sub-interface names follow the format: <interface_name>.<VLAN_ID> (e.g., G0/0.10).

- Each sub-interface:

  - Is assigned an IP address.

  - Is configured with a VLAN tag for its VLAN.

#### Diagram of ROAS Example:

### ROAS Configuration Steps:

- On the Router:

Create a sub-interface for each VLAN:

interface G0/0.10

encapsulation dot1q 10

ip address <IP_Address> <Subnet_Mask>  (last usable ip address of each subnet)

- On the Switch:

Configure the interface connected to the router as a trunk port:
switchport trunk encapsulation dot1q

  - switchport mode trunk

### Verifying ROAS Configuration:

- Check sub-interfaces with:
show ip interface brief

- View the routing table:
show ip route