# 24. DYNAMIC ROUTING

### What Is Dynamic Routing?

Dynamic routing involves configuring a dynamic routing protocol on a Layer 3 device (router) to automatically determine the best routes to destination networks. Unlike static routing, dynamic routes can adapt to changes in the network.

Key characteristics:

- Routes are not fixed and adjust dynamically.

- Routers exchange route information with neighbors to maintain up-to-date routing tables.

#### Key Terms:

💡 Network Route: A route to a network or subnet (mask length < /32).

- Example: 10.0.12.0/30, 10.0.13.0/30

💡 Host Route: A route to a specific host (mask length = /32).

- Example: 10.0.12.1/32, 10.0.13.1/32

Host routes are automatically added to a router's interfaces for connected networks.

### How Does Dynamic Routing Work?

Dynamic routing protocols allow routers to advertise network routes to their neighbors. Routers update their routing tables dynamically as networks are added or removed.

#### Example:

- Route Advertisement: R4 advertises its network to R2, which passes it to R1 and R3.

- Route Removal: If the network on R4 becomes unavailable, the route is dynamically removed from all routing tables.

Static Routing Limitation:
 In static routing, a downed route remains in the table, causing traffic to be sent to an unavailable network.

Dynamic Routing Advantage:
 Dynamic routing can automatically reroute traffic through alternative paths if redundancy is configured.

#### Example of Redundancy:

Adding a second link between R3 and R4 creates an alternative path. If the primary route via R2 fails, traffic will be rerouted via R3.

💡 Cost Factor:
 Dynamic routing protocols use a metric (e.g., cost, bandwidth, or delay) to determine the preferred path. This is similar to how spanning-tree protocols operate for switches.

### Dynamic Routing Protocols

Dynamic routing protocols enable routers to:

- Advertise information about known routes.

- Form neighbor relationships with adjacent routers.

- Decide the best route based on metrics (lower metric = better route).

#### Types of Routing Protocols:

- IGP (Interior Gateway Protocol): Shares routes within a single autonomous system (AS).

  - Examples: OSPF, EIGRP, RIP

- EGP (Exterior Gateway Protocol): Shares routes between different autonomous systems.

  - Example: BGP

#### Protocol Algorithms:

Each protocol uses a specific algorithm to calculate the best route:

- Distance Vector Protocols(Routing by Rumor): Routes by sharing only the distance (metric) and vector (direction/next hop). Examples: RIP, EIGRP.

- Link-State Protocols: Builds a complete map of the network for route calculation. Examples: OSPF, IS-IS.

### Metrics in Dynamic Routing Protocols

The metric determines the best route when multiple paths exist. Each protocol uses different metrics:

- RIP: Hop count. -> Worst

- EIGRP: Bandwidth and delay.

- OSPF: Cost (based on link bandwidth).

#### Equal Cost Multi-Path (ECMP):

If two routes to the same destination have the same metric, both are added to the routing table, and traffic is load-balanced across them.

### Administrative Distance (AD)

When routes are learned from multiple protocols, Administrative Distance (AD) determines the preferred source.

- Lower AD = Higher trustworthiness.

- Default AD values (must memorize for CCNA):

  - Directly connected: 0

  - OSPF: 110

  - EIGRP: 90

  - RIP: 120

  - External BGP: 20

💡 If the AD is 255, the route is not trusted and is not added to the routing table.

#### Example:

If OSPF and RIP both advertise a route to the same destination:

- OSPF is chosen (lower AD = 110 vs. 120).

### Floating Static Routes

A floating static route has an AD higher than the dynamic protocol’s AD, making it less preferred. It acts as a backup route and becomes active only if the dynamic route fails.

### Link-State Routing Protocols

- Routers create a complete network map by sharing detailed link-state information.

- Use more resources (e.g., CPU, memory).

- React faster to network changes than distance vector protocols.

### Summary of Key Points:

- Dynamic Routing: Adapts automatically to network changes.

- Protocols: Use metrics to determine the best route.

- Administrative Distance: Decides route preference between protocols.

- Redundancy: Ensures high availability in the network.

Trace route: Every hop sends a message back to the source