**studied less in depth than OSPF for CCNA**

# 25. RIP and EIGRP (IGP: Dynamic Vector)

## Routing Information Protocol (RIP)

### Key Characteristics

- Industry Standard: A distance-vector IGP that uses Routing-By-Rumor logic to learn/share routes.

- Metric: Uses Hop Count (One Router = One Hop), disregarding bandwidth.

- Max Hop Count: 15 (Routes beyond this are considered unreachable).

### Versions

- RIPv1: Supports only classful addressing, does not include subnet masks.

- RIPv2: Supports VLSM and CIDR, includes subnet masks in advertisements.

- RIPng: Designed for IPv6.

### Message Types

- Request: Asks RIP-enabled neighbors for their routing tables.

- Response: Sends the local router's routing table to neighbors.

By default, RIP-enabled routers share their routing table every 30 seconds.

### RIPv1 Details

- Only advertises classful addresses:

  - Example:

    - 10.1.1.0/24 → 10.0.0.0 (Class A, /8 assumed).

    - 172.16.192.0/18 → 172.16.0.0 (Class B, /16 assumed).

- Does not support VLSM or CIDR.

- Broadcasts updates to 255.255.255.255.

### RIPv2 Details

- Supports VLSM and CIDR.

- Multicasts updates to 224.0.0.9 (not broadcast).

  - Broadcast vs. Multicast:

    - Broadcast delivers to all devices.

    - Multicast delivers only to specific group members.

### RIP Configuration

- network command:

  - Activates RIP on interfaces within the specified range.

  - Forms adjacencies with RIP neighbors.

  - Advertises the network prefix of the interface.

- Classful Nature:

  - Example:

    - network 10.0.0.0 assumes /8.

    - RIP activates on interfaces within 10.0.0.0/8 and advertises prefixes like 10.0.12.0/30.

- Passive Interface:

  - Stops RIP advertisements on specified interfaces without disabling prefix advertisement.

### Advertising Default Routes in RIP

- Use the default-information originate command to share default routes.

- RIP prefers the path with the least hops for default route selection.

### Useful Commands for RIP

- show ip protocols: Displays RIP configuration.

- Adjustments:

  - Maximum paths: maximum-paths <value>.

  - Administrative Distance: distance <value>.

## Enhanced Interior Gateway Routing Protocol (EIGRP)

### Key Characteristics

- Distance Vector IGP: Advanced/hybrid version, proprietary to Cisco but now open.

- Advantages:

  - Faster than RIP in network changes.

  - No hop limit (unlike RIP's 15).

  - Supports unequal-cost load-balancing.

- Multicast Address: 224.0.0.10 (memorize this).

### EIGRP Configuration

- router eigrp :

  - AS numbers must match for adjacency formation.

  - Disable auto-summary if enabled.

- Wildcard Masks:

  - Inverted subnet masks.

  - 0 = Bits must match.

  - 1 = Bits can vary.

- Network Command:

  - Similar to RIP but uses wildcard masks.

### Metrics

- Calculated using Bandwidth and Delay.

- Default K values:

  - K1 = 1, K2 = 0, K3 = 1, K4 = 0, K5 = 0.

- Simplified Metric Formula:

  - Metric = Bandwidth (slowest link) + Delay (sum of all links).

### EIGRP Terminology

- Feasible Distance (FD): Router's metric to a destination.

- Reported Distance (RD): Neighbor's metric to a destination.

- Successor: Best route (lowest metric).

- Feasible Successor: Backup route meeting the feasibility condition.

### Feasibility Condition

- A route is a feasible successor if its RD < FD of the successor.

### Unequal-Cost Load-Balancing

- Variance command adjusts the threshold for load-balancing.

- Only feasible successors are considered for unequal-cost load-balancing.

### Key Commands

- show ip protocols: Displays EIGRP settings.

- Adjustments:

  - Variance: variance <value>.

  - Distance: Internal = 90, External = 170.