## What is a Networking Model?

Networking models categorize and provide a structure for networking protocols and standards.

(Protocols are a set of logical rules defining how network devices and software should work.)

## OSI MODEL

- Open Systems Interconnection Model

- Conceptual model that categorizes and standardizes the different functions in a network.

- Created by the "International Organization for Standardization" (ISO)

- Functions are divided into 7 "Layers."

- These layers work together to make the network work.

As data moves from the top layer, downward, the process is called encapsulation.

As data moves from the bottom layer, upward, the process is called de-encapsulation.

When interactions occur on the same layer, it’s called same-layer interaction.

Mnemonic to help remember the Data Layer Names / Order:

### The Layers Are:

### 7 - APPLICATION

- Closest layer to the end user.

- Interacts with software applications.

- HTTP and HTTPS are Layer 7 protocols.

Functions of Layer 7 include:

- Identifying communication partners

- Synchronizing communication

### 6 - PRESENTATION

- Translates data to the appropriate format (between Application and Network formats) to be sent over the network.

### 5 - SESSION

- Controls dialogues (sessions) between communicating hosts.

- Establishes, manages, and terminates connections between local application and the remote application.

Note: Network engineers don't usually work with the top three layers.
 Application developers work with these top layers to connect applications over networks.

### 4 - TRANSPORT

- Segments and reassembles data for communication between end hosts.

- Breaks large data pieces into smaller segments, reducing transmission issues if errors occur.

- Provides HOST-TO-HOST (end-to-end) communication.

When Data from Layers 7-5 arrives, it receives a Layer 4 Header in the Transport layer:

<< DATA + L4 Header >>

This is called a SEGMENT.

### 3 - NETWORK

- Provides connectivity between end hosts on different networks (e.g., outside the LAN).

- Provides logical addressing (IP Addresses).

- Provides path selection between source and destination.

- ROUTERS operate at Layer 3.

When Data and the Layer 4 Header arrive in the Network Layer, it receives a Layer 3 Header:

<< DATA + L4 Header + L3 Header >>

This is called a PACKET.

### 2 - DATA LINK

- Provides NODE-TO-NODE connectivity and data transfer (e.g., PC to Switch, Switch to Router).

- Formats data for transmission over physical medium (e.g., copper UTP cables).

- Detects and (possibly) corrects Physical (Layer 1) errors.

- Uses Layer 2 addressing, separate from Layer 3 addressing.

- SWITCHES operate at Layer 2.

When the Layer 3 Packet arrives, a Layer 2 Trailer and Header are added:

<< L2 Trailer + DATA + L4 Header + L3 Header + L2 Header >>

This is called a FRAME.

All the steps leading up to transmission are called ENCAPSULATION.
 When the frame is sent to the receiver, it goes through DE-ENCAPSULATION—stripping off layers while traveling from Layer 1 to Layer 7.

### 1 - PHYSICAL

- Defines physical characteristics of the medium used to transfer data between devices (e.g., voltage levels, connectors, cable specs).

- Converts digital bits into electrical (wired) or radio (wireless) signals.

- All of the information in Section 2 (Networking Devices) relates to the Physical Layer.

## OSI MODEL - PDUs

A PDU is a Protocol Data Unit. Each step of the process is a PDU:

| OSI Layer | PDU Name | Protocol Data Added |
| --- | --- | --- |
| 7-5 | DATA | Data |
| 4 | SEGMENT | Layer 4 Header Added |
| 3 | PACKET | Layer 3 Header Added |
| 2 | FRAME | Layer 2 Trailer + Header |
| 1 | BIT | Transmission as 0s and 1s |

## TCP/IP Suite

- Conceptual model and set of communications protocols used in the Internet and other networks.

- Known as TCP/IP because those are two foundational protocols in the suite.

- Developed by the US Dept. of Defense via DARPA (Defense Advanced Research Projects Agency).

- Similar structure to the OSI Model, but fewer layers.

- This is the model actually in use in modern networks.

- Note: The OSI Model still influences how network engineers think and talk about networks.

### Layer Interactions

Adjacent-Layer Interactions

- Interactions between different layers of the OSI Model on the same host.

Example: Layers 5-7 sending data to Layer 4, which then adds a Layer 4 header (creating a SEGMENT).

Same-Layer Interactions

- Interactions between the same layer on different hosts.

- This allows you to focus on the interaction of a single layer across devices.

Example: The Application Layer of YouTube's web server and your PC's browser.