# 41. SYSLOG

## SYSLOG OVERVIEW

- SYSLOG is an industry-standard protocol for message logging.

- On network devices, SYSLOG can be used to log events, such as:

  - Changes in interface status (UP / DOWN)

  - Changes in OSPF neighbor status (UP / DOWN)

  - System restarts

  - Other significant network events

- Messages can be displayed in the CLI, saved in the device’s RAM, or sent to an external SYSLOG server.

- Logs are essential for troubleshooting issues, examining incident causes, and monitoring system health.

- SYSLOG and SNMP are complementary tools for monitoring and troubleshooting devices, but they serve different purposes.

## SYSLOG MESSAGE FORMAT

Format:
 seq: timestamp: %facility-severity-MNEMONIC: description

💡 Note: These two fields may or may not be displayed, depending on the device’s configuration.

- seq – A sequence number indicating the order of messages.

- timestamp – A timestamp indicating when the message was generated.

- facility – Identifies which process on the device generated the message.

- severity – A numerical value indicating the severity of the event.

### SYSLOG Severity Levels (RFC Standard)

| Level | Keyword | Description |
| --- | --- | --- |
| 0 | Emergency | System is unusable |
| 1 | Alert | Immediate action required |
| 2 | Critical | Critical condition |
| 3 | Error | Error condition |
| 4 | Warning | Warning condition |
| 5 | Notice | Normal but significant event |
| 6 | Informational | Informational messages |
| 7 | Debug | Debugging messages |

💡 Mnemonic for memorization: (E)very (A)wesome (C)isco (E)ngineer (W)ill (N)eed (I)ce cream (D)aily

- MNEMONIC – A short code indicating what happened.

- description – Detailed information about the event being reported.

## SYSLOG LOGGING LOCATIONS

SYSLOG messages can be stored in different locations:

- Console Line:

  - Messages are displayed in the CLI when connected via the console port.

  - By default, all messages (Levels 0-7) are displayed.

- Buffer:

  - Messages are stored in RAM.

  - By default, all messages (Levels 0-7) are stored.

- VTY Lines:

  - Messages are displayed when connected via Telnet/SSH.

  - Disabled by default.

- External Server:

  - Messages are sent to an external SYSLOG server for central logging.

SYSLOG servers listen for messages on UDP port 514.

Good to Know:

## SYSLOG CONFIGURATION

- Logging Level: Works from the chosen level upward to Level 0 (Emergency).

- Severity Level Keywords: You can specify severity levels using keywords from the severity table.

### Terminal Monitor

- Even if logging monitor level is enabled, SYSLOG messages will not be displayed when connected via Telnet/SSH.

To display messages, use:
 R1# terminal monitor

- This command must be used each time you connect via Telnet or SSH.

### Logging Synchronous

- By default, logging messages interrupt CLI commands, causing messy output.

To prevent this, enable logging synchronous:
R1(config)# line console 0

R1(config-line)# logging synchronous

- This ensures that a new line is printed if a message interrupts your typing.

### Service Timestamps & Sequence Numbers

Enable timestamps:
 R1(config)# service timestamps log datetime

Enable sequence numbers:
 R1(config)# service sequence-numbers

## SYSLOG vs. SNMP

Both SYSLOG and SNMP are used for monitoring and troubleshooting, but they have different functionalities:

### SYSLOG

- Used for message logging.

- Logs events based on facility and severity levels.

- Helps in system management, analysis, and troubleshooting.

- Messages are pushed from devices to the server.

- Limitations: The server cannot actively pull information from devices or modify variables.

### SNMP

- Used for retrieving and organizing device information.

- Can monitor:

  - IP addresses

  - Interface status

  - Temperature, CPU usage, etc.

- SNMP servers can:

  - Get – Query information from devices.

  - Set – Modify device variables remotely.