# 47. QoS (Quality of Service) - Part 2

## Classification / Marking

- The purpose of QoS is to give certain types of network traffic priority over others during congestion.

- Classification organizes network traffic (packets) into traffic classes (categories).

- Classification is fundamental to QoS:

  - To give priority to certain types of traffic, you must identify which types of traffic deserve priority.

- There are many methods of classifying traffic:

  - ACL (Access Control List): Traffic permitted by the ACL will receive specific treatment; other traffic will not.

  - NBAR (Network-Based Application Recognition): Performs deep packet inspection (DPI), looking beyond Layer 3 & Layer 4 up to Layer 7 to identify specific types of traffic.

  - Layer 2 & Layer 3 Headers: Specific fields in headers are used for traffic classification.

- Key Fields for Classification:

  - PCP (Priority Code Point) Field in the 802.1Q tag (Ethernet header) can be used to identify high/low priority traffic.

    - Only applicable when a dot1q tag is present!

  - DSCP (Differentiated Services Code Point) Field in the IP header can also be used to classify traffic.

## PCP / CoS (Class of Service)

- PCP is also known as Class of Service (CoS).

- Defined by IEEE 802.1p.

- Uses 3 bits = 8 possible values (2³ = 8).

### PCP Values:

- PCP Value 0 (Best Effort Delivery):

  - No guarantee of delivery or QoS standard adherence.

  - Regular traffic (not high priority).

- PCP Values 3 & 5:

  - IP Phones mark call signaling traffic as PCP 3.

  - Voice traffic is marked as PCP 5.

- PCP is found in the dot1q header, so it is only used over:

  - Trunk Links

  - Access Links with a Voice VLAN

## IP ToS Byte

- The Type of Service (ToS) Byte in the IP header contains 6 bits for DSCP and 2 bits for ECN (Explicit Congestion Notification).

### IP Precedence (Old System)

- Similar to PCP:

  - 6 & 7 reserved for network control traffic (e.g., OSPF messages between routers).

  - 5 = Voice

  - 4 = Video

  - 3 = Voice signaling

  - 0 = Best Effort

- Limitations:

  - Only 6 usable values, which may not be enough for some networks.

## DSCP (Current System)

- RFC 2474 (1998) defines DSCP, with later RFCs elaborating on its use.

- Advantages of DSCP over IPP:

  - More flexible traffic classification.

  - Standardized QoS markings across ISPs and enterprises.

### Standard DSCP Markings:

- Default Forwarding (DF): Best Effort Traffic (DSCP 0)

- Expedited Forwarding (EF): Low loss, latency, jitter traffic (e.g., Voice, DSCP 46)

- Assured Forwarding (AF): 12 standard values

- Class Selector (CS): 8 standard values for backward compatibility with IP Precedence

## Assured Forwarding (AF)

- Defines 4 traffic classes, each with 3 drop precedence levels.

  - Higher drop precedence = Higher chance of being dropped during congestion.

- Examples:

  - AF41: Highest priority, lowest drop

  - AF13: Lowest priority, highest drop

## Class Selector (CS)

- Defines 8 DSCP values for backward compatibility with IP Precedence.

- The 3 additional DSCP bits are set to 0, and the original IP Precedence bits remain unchanged.

## RFC 4954 (Standardization of QoS Markings)

- Developed with Cisco to standardize QoS values.

- Key recommendations:

  - Voice Traffic: EF

  - Interactive Video: AF4x

  - Streaming Video: AF3x

  - High-Priority Data: AF2x

  - Best Effort Traffic: DF

## Trust Boundaries

- Defines where network devices trust or modify QoS markings.

- Trusted markings: Passed without modification.

- Untrusted markings: Changed according to policy.

- Recommended practice: Move the trust boundary to IP Phones using switch port configurations.

- Example:

  - PC traffic with high priority marking is modified to prevent misuse.

## Queuing & Congestion Management

- When traffic enters a network device faster than it can be forwarded, packets wait in queues.

- Full queues lead to packet drops (Tail Drop).

- RED (Random Early Detection) & WRED (Weighted RED) mitigate tail drop.

### Queue Scheduling:

- Multiple queues improve QoS.

- Scheduling determines which queue’s traffic is forwarded next.

- Prioritization: Some queues get higher priority.

### Weighted Round-Robin (WRR) Scheduling:

- Round-Robin: Packets are taken from queues in order.

- Weighted: High-priority queues get more bandwidth.

### CBWFQ (Class-Based Weighted Fair Queuing):

- Popular scheduling method.

- Uses WRR scheduler while guaranteeing each queue a specific bandwidth percentage during congestion.

## LLQ (Low Latency Queuing)

- Designates one or more queues as strict priority queues.

- If there is traffic in the queue, the scheduler will always take the next packet from that queue until it is empty.

- Very effective for reducing delay and jitter in voice/video traffic.

- Downside:

  - Other queues may be starved if there is always traffic in the strict priority queue.

  - Policing can control the amount of traffic allowed in the strict priority queue so it doesn't consume all the link’s bandwidth.

## Traffic Shaping & Policing

Both traffic shaping and policing are used to control the rate of traffic.

### Traffic Shaping

- Buffers traffic in a queue if the traffic rate exceeds the configured rate.

### Traffic Policing

- Drops traffic if the traffic rate exceeds the configured rate.

- Alternative to dropping:

  - Can re-mark traffic instead of dropping it.

  - Allows burst traffic over the configured rate for a short period.

  - Useful for data applications, which typically send data in bursts rather than a constant stream.

  - The amount of burst traffic allowed is configurable.

### Classification in Both Cases

- Different rates can be applied to different types of traffic.

### Why Limit Traffic Rates?

- To prevent congestion and ensure fair bandwidth distribution.