# 49. PORT SECURITY

## INTRODUCTION TO PORT SECURITY

- Port Security is a security feature of Cisco switches.

- It allows you to control which source MAC address(es) are allowed to enter the switch port.

- If an unauthorized source MAC address enters the port, an action will be taken:

  - The default action is to place the interface in an “err-disabled” state.

- When you enable Port Security on an interface with default settings, one MAC address is allowed.

  - You can configure the allowed MAC address manually.

  - If you do not configure it manually, the switch will allow the first source MAC address that enters the interface.

- You can change the maximum number of MAC addresses allowed.

- A combination of manually configured MAC addresses and dynamically learned addresses is possible.

## WHY USE PORT SECURITY?

- Port Security allows network administrators to control which devices are allowed to access the network.

- However, MAC address spoofing is a simple task:

  - It is easy to configure a device to send frames with a different source MAC address.

- Instead of manually specifying allowed MAC addresses, limiting the number of MAC addresses per interface is more practical.

- Example: DHCP Starvation Attack (Day 48 Lab Video):

  - The attacker spoofed thousands of fake MAC addresses.

  - The DHCP server assigned IP addresses to these fake MAC addresses, exhausting the DHCP pool.

  - The switch’s MAC address table can also become full due to such an attack.

- Limiting the number of MAC addresses per interface can help prevent these attacks.

## ENABLING PORT SECURITY

### Show Port Security Information:

SW1#show port-security interface g0/1

### Re-Enabling an Interface (Manually)

SW1#int g0/1

SW1(config-if)#shutdown

SW1(config-if)#no shutdown

### Re-Enabling an Interface (Err-Disable Recovery)

## VIOLATION MODES

There are three different violation modes that determine what the switch will do if an unauthorized frame enters an interface with Port Security enabled:

- Shutdown

  - Shuts down the port by placing it in an err-disabled state.

  - Generates a SYSLOG and/or SNMP message when the interface is disabled.

  - The violation counter is set to 1 when the interface is disabled.

- Restrict

  - The switch discards traffic from unauthorized MAC addresses.

  - The interface is NOT disabled.

  - Generates a SYSLOG and/or SNMP message each time an unauthorized MAC is detected.

  - The violation counter increments by 1 for each unauthorized frame.

- Protect

  - The switch discards traffic from unauthorized MAC addresses.

  - The interface is NOT disabled.

  - Does NOT generate SYSLOG/SNMP messages for unauthorized traffic.

  - Does NOT increment the violation counter.

### Violation Mode - Restrict

### Violation Mode - Protect

## SECURE MAC ADDRESS AGING

- By default, secure MAC addresses will not age out (Aging Time: 0 minutes).

  - Can be configured using: switchport port-security aging time *minutes*

- Default Aging Type: Absolute

  - Absolute: After a secure MAC address is learned, the aging timer starts and the MAC is removed after the timer expires, even if frames are still received from that source.

  - Inactivity: After a secure MAC address is learned, the aging timer resets every time a frame is received from that source.

    - Configure with: switchport port-security aging type {absolute | inactivity}

- Secure Static MAC Aging is disabled by default.

## STICKY SECURE MAC ADDRESSES

- Sticky secure MAC address learning can be enabled with:

  - SW(config-if)# switchport port-security mac-address sticky

- When enabled, dynamically-learned secure MAC addresses are added to the running configuration:

  - switchport port-security mac-address sticky *mac-address*

- Sticky MAC addresses never age out.

  - To make them permanent, save the running-config to startup-config.

- Issuing switchport port-security mac-address sticky converts all current dynamically-learned MAC addresses to sticky.

- Issuing no switchport port-security mac-address sticky converts all sticky MAC addresses back to dynamically learned.

## MAC ADDRESS TABLE

- Secure MAC addresses are added to the MAC address table:

  - Sticky and static secure MAC addresses are labeled STATIC.

  - Dynamically-learned secure MAC addresses are labeled DYNAMIC.

  - View all secure MAC addresses with:

    - show mac address-table secure

## COMMAND REVIEW