# 5. ETHERNET LAN SWITCHING: PART 1

### What is a LAN?

- A LAN (Local Area Network) is a network contained in a relatively small area.

- Routers are used to connect separate LANs.

### Structure of an Ethernet Frame

Ethernet Frame:

- Ethernet Header | Packet | Ethernet Trailer

#### Ethernet Header Fields (26 bytes total):

- Preamble:

  - Length: 7 bytes (56 bits).

  - Alternating 1s and 0s: 10101010 (repeated 7 times).

  - Allows devices to synchronize receiver clocks.

- SFD (Start Frame Delimiter):

  - Length: 1 byte (8 bits).

  - Value: 10101011.

  - Marks the end of the preamble and start of the frame.

- Destination and Source Address:

  - Layer 2 Address (MAC Address).

  - 6 bytes (48 bits) each for source and destination.

  - Represents the physical address of devices.

- Type/Length Field:

  - Length: 2 bytes (16 bits).

  - Determines either:

    - Length of the encapsulated packet (if ≤ 1500).

    - Type of the encapsulated packet (if ≥ 1536).

  - Examples:

    - IPv4 = 0x0800 (2048 in decimal).

    - IPv6 = 0x86DD (34525 in decimal).

#### Ethernet Trailer Field

- FCS (Frame Check Sequence):

  - Length: 4 bytes (32 bits).

  - Used for error detection via CRC (Cyclic Redundancy Check).

  - Detects corrupted data in the frame.

### MAC Address

- Length: 6 bytes (48 bits).

- Also known as the Burned-In Address (BIA).

- Globally unique for every device.

- Structure:

  - First 3 bytes: OUI (Organizationally Unique Identifier).

  - Last 3 bytes: Unique to the device.

- Represented in hexadecimal: e.g., E8:BA:70:11:28:74.

### Interface Names

- Common format: F0/1, F0/2, F0/3...

  - F: Indicates Fast Ethernet (100 Mbps interfaces).

### MAC Address Table

Dynamic MAC Address Table:

- A switch learns MAC addresses dynamically using the Source MAC Address from incoming frames.

- Stored MAC addresses allow efficient frame forwarding to the correct destination.

#### Frame Forwarding Logic

- Unknown Unicast Frame:

  - When the Destination MAC Address is not found in the MAC Address Table.

  - The frame is flooded: sent out on all interfaces except the one it was received on. Only devices that sent out frames will have their MAC Address learnt in the table.

- Known Unicast Frame:

  - If the Destination MAC Address is recognized, the frame is forwarded to the appropriate interface.

#### MAC Address Table Timeout

- Dynamic MAC addresses are removed from the MAC Address Table after 5 minutes of inactivity.