# 51. Dynamic ARP Inspection (DAI)

## What is Dynamic ARP Inspection (DAI)?

### ARP Review

- ARP is used to learn the MAC address of another device with a known IP address.

  - For example, a PC will use ARP to learn the MAC address of its default gateway to communicate with external networks.

- Typically, it is a two-message exchange:

  - ARP Request

  - ARP Reply

### Gratuitous ARP (GARP)

- A Gratuitous ARP (GARP) message is an ARP reply that is sent without receiving an ARP request.

- It is sent to the broadcast MAC address.

- It allows other devices to learn the MAC address of the sending device without having to send ARP requests.

- Some devices automatically send GARP messages when:

  - An interface is enabled.

  - An IP address is changed.

  - A MAC address is changed.

## Dynamic ARP Inspection (DAI)

- DAI is a security feature of switches that filters ARP messages received on untrusted ports.

- DAI only filters ARP messages; non-ARP messages are not affected.

- By default, all ports are untrusted.

  - Typically, all ports connected to other network devices (switches, routers) should be configured as trusted.

  - Interfaces connected to end hosts should remain untrusted.

## ARP Poisoning (Man-in-the-Middle Attack)

- Similar to DHCP poisoning, ARP poisoning involves an attacker manipulating a target’s ARP tables so that traffic is sent to the attacker.

- The attacker can send Gratuitous ARP messages using another device’s IP address.

- Other devices in the network will receive the GARP and update their ARP tables, causing them to send traffic to the attacker instead of the legitimate destination.

## Dynamic ARP Inspection Operations

- DAI inspects the Sender MAC and Sender IP fields of ARP messages received on untrusted ports and checks for a matching entry in the DHCP Snooping Binding Table:

  - If there is a match, the ARP message is forwarded.

  - If there is no match, the ARP message is discarded.

- DAI does not inspect messages received on trusted ports; they are forwarded as normal.

- ARP ACLs can be manually configured to map IP addresses to MAC addresses for DAI verification.

  - Useful for hosts that don’t use DHCP.

- DAI can be configured for additional checks, though these are optional.

- Rate-limiting is supported to prevent attackers from overwhelming the switch with ARP messages.

  - Both DHCP Snooping and DAI require processing by the switch’s CPU.

  - Even if the attacker's messages are blocked, they can overload the switch CPU with ARP messages.

## Dynamic ARP Inspection Configuration

### Basic Commands:

Verify configuration:
 SW1#show ip arp inspection interfaces

Enable DAI on a VLAN:
 SW2(config)# ip arp inspection vlan <VLAN_ID>

Set a trusted interface:
 SW2(config)# interface <INTERFACE>

SW2(config)# ip arp inspection trust

Configure rate-limiting:
 SW1# ip arp inspection limit rate <RATE>

Check DAI statistics:
 SW1#show ip arp inspection

## ARP ACLs (Beyond Scope of CCNA)

- ARP ACLs can be used to define static mappings for IP and MAC addresses.

Example: Creating an ARP ACL for SRV1
 ip arp access-list <ACL_NAME>

permit ip host <IP_ADDRESS> mac host <MAC_ADDRESS>

Apply ACL to a switch:
 ip arp inspection filter <ACL_NAME> vlan <VLAN_ID>

## Command Review

| Command | Description |
| --- | --- |
| show ip arp inspection | Displays a summary of DAI configuration and statistics |
| ip arp inspection vlan <VLAN_ID> | Enables DAI on a specific VLAN |
| ip arp inspection trust | Configures an interface as trusted |
| ip arp inspection limit rate <RATE> | Limits the rate of ARP messages |
| show ip arp inspection interfaces | Displays per-interface DAI settings |